## 🔦 G6S11 – Robotics Project

# Smart LED using Motion Sensor (PIR-Activated Light)

This project uses a PIR (Passive Infrared) Motion Sensor to detect human movement and automatically turn ON an LED. When no motion is detected, the LED turns OFF.

## 🧠 How It Works

- PIR sensor detects infrared radiation from the human body.

- When motion is detected → PIR output becomes HIGH

- Arduino reads HIGH signal → LED turns ON 💡

- When no motion → Output becomes LOW → LED turns OFF

## 🛠️ Components Required

- Arduino UNO

- PIR Motion Sensor (HC-SR501)

- LED

- 220Ω Resistor

- Breadboard

- Jumper Wires

## 🔌 Circuit Connections

### PIR Sensor → Arduino

- VCC → 5V

- GND → GND

- OUT → Pin 2

### LED → Arduino

- LED Long Leg (+) → Pin 13

- LED Short Leg (–) → GND (via 220Ω resistor)

## 💻 Arduino Code

int pirPin = 2;

int ledPin = 13;

int motionState = 0;

void setup() {

pinMode(pirPin, INPUT);

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

motionState = digitalRead(pirPin);

if (motionState == HIGH) {

digitalWrite(ledPin, HIGH);

Serial.println("Motion Detected!");

} else {

digitalWrite(ledPin, LOW);

Serial.println("No Motion");

}

delay(500);

}

## ⚙️ PIR Sensor Adjustment

The PIR module has:

- Sensitivity Control (Distance Adjustment)

- Time Delay Control (How long LED stays ON)

Rotate the small blue potentiometers to adjust.

## 🎯 Applications

- Automatic Room Light

- Security Alarm System

- Smart Street Light

- Energy Saving System

## 🔥 If You Want Upgrade Version:

You can add:

- Buzzer 🔔

- Relay module (to control real bulb)

- LCD display

- IoT control

If you want, I can also give you:

- 📄 Viva Questions & Answers

- 📝 Project Report (Ready to Print PDF)

- 📊 Block Diagram & Flowchart

Tell me what you need 👍