Badhiya project choose kiya hai 🔥
G12S19 – IoT ESP8266 SoftAP Web Dashboard (Live DHT) exam + practical dono ke liye perfect hai.
Main bilkul scratch se step-by-step samjha raha hoon, jaise lab file + demo dono clear ho jaaye.

## 📌 Project Objective (Simple words)

ESP8266:

- khud ka Wi-Fi hotspot (SoftAP) banayega

- Mobile/Laptop us Wi-Fi se connect hoga

- Browser me local web dashboard open hoga

- Dashboard par DHT11/DHT22 ka live Temperature & Humidity dikhega

- Internet ki bilkul zarurat nahi

## 🧠 Concept samjho (Exam me yahi puchte hain)

| Term | Meaning |
| --- | --- |
| ESP8266 SoftAP | ESP8266 apna Wi-Fi banata hai |
| Local Web Server | ESP browser ko HTML page bhejta hai |
| DHT Sensor | Temperature + Humidity sensor |
| Live Data | Page auto-refresh / AJAX se update |

## 🔧 Required Components

| Item | Quantity |
| --- | --- |
| ESP8266 NodeMCU | 1 |
| DHT11 / DHT22 | 1 |
| 10kΩ Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | Few |
| USB Cable | 1 |

## 🔌 Circuit Connections

### ✅ DHT11 → ESP8266

| DHT Pin | ESP8266 |
| --- | --- |
| VCC | 3.3V |
| DATA | D4 (GPIO2) |
| GND | GND |
| DATA → VCC | 10kΩ resistor |

⚠️ ESP8266 is 3.3V only — 5V mat dena

## 🛠️ Software Setup

### 1️⃣ Arduino IDE Settings

- Board: NodeMCU 1.0 (ESP-12E Module)

- Port: COMx

- Install Libraries:

  - ESP8266WiFi

  - ESP8266WebServer

  - DHT sensor library

  - Adafruit Unified Sensor

## 🧾 Working Logic (Flow)

ESP8266 Power ON

↓

SoftAP WiFi create karega

↓

User connect karega (Mobile/Laptop)

↓

Browser → 192.168.4.1

↓

HTML Dashboard load

↓

DHT se Temp & Humidity read

↓

Live value show

## 🧑‍💻 Complete Arduino Code

#include <ESP8266WiFi.h>

#include <ESP8266WebServer.h>

#include "DHT.h"

#define DHTPIN D4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

ESP8266WebServer server(80);

void setup() {

Serial.begin(9600);

dht.begin();

WiFi.softAP("ESP_DHT_Dashboard", "12345678");

Serial.println("SoftAP Started");

Serial.println(WiFi.softAPIP());

server.on("/", handleRoot);

server.begin();

}

void loop() {

server.handleClient();

}

void handleRoot() {

float h = dht.readHumidity();

float t = dht.readTemperature();

String html = "<html><head><title>DHT Dashboard</title>";

html += "<meta http-equiv='refresh' content='2'>";

html += "<style>body{font-family:Arial;text-align:center;}</style>";

html += "</head><body>";

html += "<h1>ESP8266 DHT Live Dashboard</h1>";

html += "<h2>Temperature: " + String(t) + " °C</h2>";

html += "<h2>Humidity: " + String(h) + " %</h2>";

html += "</body></html>";

server.send(200, "text/html", html);

}

## 📱 How to Use (Demo Steps)

- ESP8266 power ON

- Mobile Wi-Fi → ESP_DHT_Dashboard

- Password → 12345678

- Browser → 192.168.4.1

- 🎉 Live Temperature & Humidity show hoga

## 🧪 Output (Viva bolne ke points)

- ESP8266 acts as Access Point

- Web dashboard works offline

- Sensor data refresh every 2 seconds

- No cloud, no internet

## 📄 Lab File / Viva Ready Explanation

Applications:

- Smart Home monitoring

- Weather station

- Agriculture monitoring

- Classroom IoT demo

Advantages:

- No internet needed

- Low cost

- Portable

- Fast response

## 🔥 Next Level (A+ ke liye)

Chaaho to mai:

- AJAX based no refresh dashboard

- Mobile responsive UI

- Graph (chart)

- PDF Lab File + Block Diagram

- Viva questions & answers