Badhiya choice, Chandan 🔥
“ESP8266 Smart Alert (IoT Automation Basics)” project beginner-friendly bhi hai aur practical + demo-worthy bhi.
Main tumhe scratch se step-by-step bata raha hoon — bilkul lab file + viva ready style me 😄

## 🔹 Project Name

ESP8266 Smart Alert System (IoT Automation Basics)

## 🔹 Project Objective

ESP8266 Wi-Fi module ka use karke sensor detect hone par alert dena:

- LED / Buzzer ON

- Mobile / Web Dashboard par alert

- (Optional) Email / Telegram alert

## 🔹 Required Components (Hardware)

| Component | Quantity |
| --- | --- |
| ESP8266 NodeMCU | 1 |
| Sensor (choose one):• PIR (Motion) OR• Gas Sensor (MQ-2) OR• Ultrasonic Sensor | 1 |
| Buzzer | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |

👉 Best for beginners: PIR Motion Sensor
(door security / intrusion alert)

## 🔹 Software Required

- Arduino IDE

- ESP8266 Board Package

- Wi-Fi (Mobile Hotspot bhi chalega)

## 🔹 Step-1: Arduino IDE Setup (ESP8266)

- Arduino IDE → File → Preferences

- “Additional Board Manager URLs” me paste karo:

http://arduino.esp8266.com/stable/package_esp8266com_index.json

- Tools → Board → Boards Manager
→ Search ESP8266 → Install

- Select Board:

NodeMCU 1.0 (ESP-12E Module)

## 🔹 Step-2: Circuit Connections (PIR Based)

### 🔌 PIR Sensor → ESP8266

| PIR Pin | ESP8266 |
| --- | --- |
| VCC | 3V3 |
| GND | GND |
| OUT | D5 |

### 🔔 Buzzer

| Buzzer | ESP8266 |
| --- | --- |
| + | D6 |
| – | GND |

### 💡 LED

| LED | ESP8266 |
| --- | --- |
| Anode (+) | D7 (via 220Ω) |
| Cathode (–) | GND |

## 🔹 Step-3: Working Logic (Simple Words)

- PIR motion detect karega

- ESP8266 Wi-Fi se connect hoga

- Motion detect →

  - LED ON

  - Buzzer ON

  - Browser par “ALERT! Motion Detected”

## 🔹 Step-4: ESP8266 Code (Web Alert)

#include <ESP8266WiFi.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_PASSWORD";

WiFiServer server(80);

#define PIR D5

#define BUZZER D6

#define LED D7

void setup() {

Serial.begin(9600);

pinMode(PIR, INPUT);

pinMode(BUZZER, OUTPUT);

pinMode(LED, OUTPUT);

digitalWrite(BUZZER, LOW);

digitalWrite(LED, LOW);

WiFi.begin(ssid, password);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected!");

Serial.println(WiFi.localIP());

server.begin();

}

void loop() {

WiFiClient client = server.available();

int motion = digitalRead(PIR);

if (motion == HIGH) {

digitalWrite(LED, HIGH);

digitalWrite(BUZZER, HIGH);

} else {

digitalWrite(LED, LOW);

digitalWrite(BUZZER, LOW);

}

if (client) {

String request = client.readStringUntil('\r');

client.flush();

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: text/html\n");

client.println("<html><body>");

if (motion == HIGH)

client.println("<h1 style='color:red;'>ALERT! Motion Detected</h1>");

else

client.println("<h1 style='color:green;'>Safe Area</h1>");

client.println("</body></html>");

}

}

## 🔹 Step-5: How to Use

- Code upload karo

- Serial Monitor open karo (9600 baud)

- IP address copy karo

- Mobile / Laptop browser me paste karo

- Motion ke samne haath le jao → Alert show hoga

## 🔹 Output / Result

✅ Motion detection
✅ Real-time IoT alert
✅ LED + Buzzer indication
✅ Web dashboard

## 🔹 Viva Questions (Exam Ready 😎)

Q. Why ESP8266?
👉 Inbuilt Wi-Fi + low cost

Q. IoT kya hota hai?
👉 Internet ke through devices ka control & monitoring

Q. PIR sensor ka use?
👉 Motion detect karne ke liye

## 🔹 Optional Advanced Features

(Teacher impress karne ke liye 😁)

- Telegram Alert

- Email Notification

- Blynk App control

- Firebase logging

Agar tum chaho to:

- 📄 Lab file format

- 📊 Block diagram

- 📱 Mobile app version

- 🔥 Gas / Fire alert version

bas bolo — next step kaunsa chahiye? 🚀