Great 😍
Ab project banega Smart Attendance Counter (Entry + Exit) with 16×2 LCD Display

Isme:

- Entry → Count +1

- Exit → Count –1

- LCD par live number show hoga

- Serial Monitor par bhi dikhega

# 🧰 Components

- Arduino UNO

- 2 × Ultrasonic Sensors

- 16×2 LCD (without I2C)

- Jumper wires

- 10k potentiometer (LCD contrast ke liye)

# 🔌 Ultrasonic Wiring

### Entry Sensor

- VCC → 5V

- GND → GND

- TRIG → Pin 9

- ECHO → Pin 10

### Exit Sensor

- VCC → 5V

- GND → GND

- TRIG → Pin 7

- ECHO → Pin 6

# 🔌 LCD 16×2 Wiring (Without I2C)

| LCD Pin | Arduino Pin |
| --- | --- |
| VSS | GND |
| VDD | 5V |
| VO | Pot middle pin |
| RS | 12 |
| RW | GND |
| E | 11 |
| D4 | 5 |
| D5 | 4 |
| D6 | 3 |
| D7 | 2 |
| A | 5V |
| K | GND |

Potentiometer ke side pins:

- Ek side → 5V

- Dusra side → GND

# 💻 Full Code (LCD + Entry + Exit)

#include <LiquidCrystal.h>

// LCD pins

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

// Entry sensor

int trig1 = 9;

int echo1 = 10;

// Exit sensor

int trig2 = 7;

int echo2 = 6;

long duration1, duration2;

int distance1, distance2;

int count = 0;

void setup() {

lcd.begin(16, 2);

Serial.begin(9600);

pinMode(trig1, OUTPUT);

pinMode(echo1, INPUT);

pinMode(trig2, OUTPUT);

pinMode(echo2, INPUT);

lcd.print("Attendance");

lcd.setCursor(0,1);

lcd.print("Counter");

delay(2000);

lcd.clear();

}

void loop() {

// Entry sensor reading

digitalWrite(trig1, LOW);

delayMicroseconds(2);

digitalWrite(trig1, HIGH);

delayMicroseconds(10);

digitalWrite(trig1, LOW);

duration1 = pulseIn(echo1, HIGH);

distance1 = duration1 * 0.034 / 2;

// Exit sensor reading

digitalWrite(trig2, LOW);

delayMicroseconds(2);

digitalWrite(trig2, HIGH);

delayMicroseconds(10);

digitalWrite(trig2, LOW);

duration2 = pulseIn(echo2, HIGH);

distance2 = duration2 * 0.034 / 2;

// Entry detected

if (distance1 < 20 && distance1 > 0) {

count++;

updateLCD();

Serial.print("Entered: ");

Serial.println(count);

delay(2000);

}

// Exit detected

if (distance2 < 20 && distance2 > 0) {

if (count > 0) count--;

updateLCD();

Serial.print("Exited: ");

Serial.println(count);

delay(2000);

}

}

void updateLCD() {

lcd.clear();

lcd.setCursor(0,0);

lcd.print("Students Inside:");

lcd.setCursor(0,1);

lcd.print(count);

}

# 🖥️ LCD Par Kya Dikhega

Students Inside:

5

Number entry/exit ke saath change hota rahega.

# 🎤 Exhibition Me Explanation

"Gate par 2 sensors lage hain.
Ek entry count karta hai aur ek exit count karta hai.
LCD screen par class ke andar kitne students hain wo live show hota hai."

# ⚠️ Common Problems

- LCD blank ho → Potentiometer ghumaao

- Only boxes show → wiring check karo

- Count fast change ho → sensors thoda door lagao

# Next Level Upgrade 😎

Main isme add kar sakta hoon:

- 🔔 Buzzer on entry

- 🔴 LED blink

- 📊 Full attendance limit alert (like 30 students full)

Add karna hai?
Reply: Buzzer / LED / Both