## 🔧 Final Stable Setup – Temperature Logger + Alert

Components: Arduino UNO + DHT11 module (3-pin) + 16×2 LCD (without I2C) + Active buzzer + 10k potentiometer

# 🔌 Final Connections (Step-by-Step)

## 🌡 DHT11 Module (3 pins)

- VCC → 5V

- GND → GND

- DATA → Pin 2

## 🔔 Active Buzzer

  - → Pin 3

- – → GND

## 📺 16×2 LCD Connections

| LCD Pin | Arduino |
| --- | --- |
| VSS | GND |
| VDD | 5V |
| V0 | Potentiometer middle pin |
| RS | Pin 7 |
| RW | GND |
| E | Pin 8 |
| D4 | Pin 9 |
| D5 | Pin 10 |
| D6 | Pin 11 |
| D7 | Pin 12 |
| A | 5V |
| K | GND |

### Potentiometer (Contrast)

- Left pin → 5V

- Right pin → GND

- Middle pin → LCD V0

# 💻 FINAL STABLE CODE (Temp + Humidity + Alert)

Library install:

- Arduino IDE → Library Manager → install DHT sensor library

#include <LiquidCrystal.h>

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT11

#define BUZZER 3

LiquidCrystal lcd(7, 8, 9, 10, 11, 12);

DHT dht(DHTPIN, DHTTYPE);

void setup() {

lcd.begin(16, 2);

dht.begin();

pinMode(BUZZER, OUTPUT);

Serial.begin(9600);

lcd.print("Temp Logger");

delay(2000);

lcd.clear();

}

void loop() {

float temp = dht.readTemperature();

float hum = dht.readHumidity();

if (isnan(temp) || isnan(hum)) {

lcd.setCursor(0, 0);

lcd.print("Sensor Error");

return;

}

// LCD Display

lcd.setCursor(0, 0);

lcd.print("Temp:");

lcd.print(temp);

lcd.print((char)223); // degree symbol

lcd.print("C ");

lcd.setCursor(0, 1);

lcd.print("Hum:");

lcd.print(hum);

lcd.print("% ");

// Serial Monitor Log

Serial.print("Temp: ");

Serial.print(temp);

Serial.print("  Hum: ");

Serial.println(hum);

// 🔔 Alert Condition

if (temp > 35) {

digitalWrite(BUZZER, HIGH);

delay(300);

digitalWrite(BUZZER, LOW);

delay(300);

} else {

digitalWrite(BUZZER, LOW);

delay(1500);

}

}

# 🧪 Output

LCD par show hoga:

Temp:28°C

Hum:65%

- 35°C se upar → buzzer beep-beep

- Normal temp → sirf display

# ⚠️ Troubleshooting

- LCD blank → potentiometer ghumao

- 0°C ya error → DHT11 DATA pin check

- Buzzer silent → +/– ulta na laga ho

# 🎯 Project Demo Line (School/College)

“Ye system room temperature aur humidity monitor karta hai aur 35°C se upar alert deta hai.”