Great 👍
Aapke paas 4-pin sound sensor module hai, to wiring aur easy ho jayegi. Isme usually ye pins hoti hain:

- VCC

- GND

- DO (Digital Output)

- AO (Analog Output)

Is project me hum DO pin use karenge (clap/sound detect ke liye).

## 🔌 Final Connection (Light + Sound Alarm)

### 🎤 Sound Sensor (4-Pin)

- VCC → 5V

- GND → GND

- DO → D2 (Arduino)

- AO → Not needed

### 🌞 LDR (simple LDR, not module)

- LDR ka ek pin → 5V

- Dusra pin → A0

- A0 se 10k resistor → GND

### 🔔 Buzzer

  - → D8

- – → GND

## 💻 Final Working Code

int ldrPin = A0;

int soundPin = 2;   // DO pin connected

int buzzer = 8;

int lightValue;

int soundValue;

void setup() {

pinMode(soundPin, INPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

lightValue = analogRead(ldrPin);

soundValue = digitalRead(soundPin);

Serial.print("Light: ");

Serial.print(lightValue);

Serial.print("  Sound: ");

Serial.println(soundValue);

// Agar andhera ho ya light suddenly change ho

if (lightValue < 300) {

digitalWrite(buzzer, HIGH);

}

// Agar clap/sound detect ho

else if (soundValue == HIGH) {

digitalWrite(buzzer, HIGH);

}

else {

digitalWrite(buzzer, LOW);

}

delay(200);

}

## ⚙️ Important Setting (Very Important)

Sound sensor pe ek chhota blue screw (potentiometer) hota hai:

- Clockwise ghumao → sensitivity badhegi

- Clap karo aur dekho LED blink hoti hai ya nahi

- LED blink = sound detect ho raha hai

## 🧪 Testing Steps

- Arduino ON karo

- LDR ko haath se cover karo → buzzer bajna chahiye

- Clap karo → buzzer bajna chahiye

## Problem Fix Guide

- Buzzer always ON → sound sensor ka knob adjust karo

- Clap pe react nahi karta → aur tight adjust karo

- Light pe react nahi karta → threshold 300 ko 400/500 karo