Rain Drop Detection + Servo Motor Project with Arduino. When the rain sensor detects water drops, the servo motor will rotate 90° (for example to close a window or roof). When there is no rain, the servo goes back to 0°.

I will explain components, connection, and code step-by-step.

## 1. Components Required

- Arduino Uno

- Rain Drop Sensor Module

- Servo Motor (SG90)

- Jumper Wires

- Breadboard (optional)

## 2. Connection Diagram

### Rain Sensor → Arduino

- VCC → 5V

- GND → GND

- DO → Pin 7

### Servo Motor → Arduino

- Red → 5V

- Brown/Black → GND

- Orange/Yellow → Pin 9

## 3. Working Logic

- Rain sensor checks for water drops.

- If water is detected → Arduino sends signal to servo.

- Servo rotates 90°.

- If no rain → Servo returns to 0°.

## 4. Arduino Code

#include <Servo.h>

Servo myServo;

int rainSensor = 7;

int rainValue;

void setup() {

pinMode(rainSensor, INPUT);

myServo.attach(9);

myServo.write(0);   // initial position

Serial.begin(9600);

}

void loop() {

rainValue = digitalRead(rainSensor);

if (rainValue == LOW) {   // water detected

Serial.println("Rain Detected");

myServo.write(90);

}

else {                    // no rain

Serial.println("No Rain");

myServo.write(0);

}

delay(500);

}

## 5. Testing

- Upload the code.

- Open Serial Monitor.

- Put water drops on the sensor plate.

- Servo will rotate 90°.

## 6. Real Life Use

This project can be used for:

- Automatic Window Closer

- Smart Clothes Drying System

- Automatic Roof System

- Rain Alarm

✅ Since you are already making many Arduino sensor projects, I can also show you a more impressive version for project exhibition:

- Rain sensor

- Servo motor

- 16×2 LCD with I2C (to display “Rain Detected”)

- Buzzer alarm

It will look much more professional for college projects.

If you want, I can also give:

- Complete circuit diagram

- Advanced code with LCD + buzzer.