## 🌞 Solar Tracker Idea

A solar tracker moves a solar panel toward the direction where sunlight is strongest so the panel gets maximum energy.

You will use:

- 2 LDR sensors → detect light difference

- Arduino Uno → decision making

- Servo motor → rotate solar panel

# 🧰 Components Required

- Arduino Uno

- 2 LDR modules

- 1 Servo Motor (SG90 recommended)

- Breadboard

- Jumper wires

- Small Solar panel (optional for demo)

# ⚡ Connection Diagram

### LDR Module Connections

LDR 1

- VCC → 5V (Arduino)

- GND → GND

- AO → A0

LDR 2

- VCC → 5V

- GND → GND

- AO → A1

### Servo Motor

- Red → 5V

- Brown/Black → GND

- Orange/Yellow → Pin 9

# 🧠 Working Logic

- Arduino reads light values from both LDRs.

- If left LDR > right LDR, servo moves left.

- If right LDR > left LDR, servo moves right.

- When both are equal → servo stops.

# 💻 Arduino Code

#include <Servo.h>

Servo myservo;

int ldrLeft = A0;

int ldrRight = A1;

int servoPos = 90;

void setup() {

myservo.attach(9);

myservo.write(servoPos);

Serial.begin(9600);

}

void loop() {

int leftValue = analogRead(ldrLeft);

int rightValue = analogRead(ldrRight);

Serial.print("Left: ");

Serial.print(leftValue);

Serial.print(" Right: ");

Serial.println(rightValue);

if (leftValue > rightValue + 50) {

servoPos--;

myservo.write(servoPos);

delay(20);

}

if (rightValue > leftValue + 50) {

servoPos++;

myservo.write(servoPos);

delay(20);

}

delay(100);

}

# 🏗️ Physical Setup

Place the two LDRs like this:

LDR1 | divider | LDR2

Put a small wall between them so each senses light from different directions.

# 🎯 Result

- If sunlight comes from left → panel turns left

- If sunlight comes from right → panel turns right