You can easily make a Gas Detection System using MQ-2 Gas Sensor + Arduino UNO.
When gas is detected → Red LED + Buzzer ON
When no gas → Green LED ON.

## 1️⃣ Components Required

- Arduino UNO

- MQ-2 Gas Sensor

- Red LED

- Green LED

- Buzzer

- 2 × 220Ω Resistor

- Breadboard

- Jumper Wires

## 2️⃣ Pin Connection

### MQ-2 Sensor

| MQ2 Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| DO | D2 |

(We are using Digital Output)

### LEDs

| Component | Arduino |
| --- | --- |
| Red LED (+) | D3 |
| Red LED (-) | GND (via 220Ω) |
| Green LED (+) | D4 |
| Green LED (-) | GND (via 220Ω) |

### Buzzer

| Buzzer Pin | Arduino |
| --- | --- |
| + | D5 |
| – | GND |

## 3️⃣ Arduino Code

int gasSensor = 2;

int redLED = 3;

int greenLED = 4;

int buzzer = 5;

void setup() {

pinMode(gasSensor, INPUT);

pinMode(redLED, OUTPUT);

pinMode(greenLED, OUTPUT);

pinMode(buzzer, OUTPUT);

}

void loop() {

int gasValue = digitalRead(gasSensor);

if (gasValue == LOW)  // Gas detected

{

digitalWrite(redLED, HIGH);

digitalWrite(greenLED, LOW);

digitalWrite(buzzer, HIGH);

}

else  // No gas

{

digitalWrite(redLED, LOW);

digitalWrite(greenLED, HIGH);

digitalWrite(buzzer, LOW);

}

}

## 4️⃣ Working

- MQ2 detects LPG / Smoke / Gas

- Sensor DO pin becomes LOW

- Arduino turns Red LED + Buzzer ON

- When gas is not detected:

  - Green LED ON

  - Red LED and Buzzer OFF

✅ Tip:
After powering the MQ-2 sensor, wait 20–30 seconds for calibration.

👍 If you want, I can also show you:

- Full circuit diagram (very easy)

- How to adjust MQ-2 sensitivity

- How to display Gas Status on LCD (I2C)
which will make your project look more advanced for exhibition.