You want:
When IR sensor detects an object → Servo rotates to 20° → stays open for 1 second → then closes back to 0°.

### 🔌 Connection

IR Sensor

- VCC → 5V

- GND → GND

- OUT → Pin 2

Servo Motor

- Red → 5V

- Brown/Black → GND

- Yellow/Orange → Pin 9

### 💻 Arduino Code

#include <Servo.h>

Servo myServo;

int irPin = 2;

int irState = 0;

void setup() {

pinMode(irPin, INPUT);

myServo.attach(9);

myServo.write(0);   // initial position

}

void loop() {

irState = digitalRead(irPin);

if (irState == LOW) {   // object detected

myServo.write(20);    // open servo

delay(1000);          // keep open for 1 second

myServo.write(0);     // close servo

delay(1000);          // small delay to avoid repeat

}

}

### ⚙️ Working

- IR sensor detects object.

- Servo rotates to 20°.

- Waits 1 second.

- Servo returns to 0°.

💡 If you want, I can also give a very good mini project idea using this code like:

- Automatic Smart Dustbin (IR + Servo)

- Automatic Door System

- Touchless Sanitizer Machine.