Great idea Chandan 👍 You can build a Complete Smart Home Automation System using these sensors and show all status on an OLED display.

Your system will work like this:

| Sensor | Action |
| --- | --- |
| MQ2 Gas Sensor | Gas detected → Buzzer ON |
| IR Motion Sensor | Motion detected → Light ON |
| Rain Sensor | Rain detected → Servo closes window |
| LDR Sensor | Dark room → Light ON automatically |
| DHT11 | Show Temperature & Humidity |
| Ultrasonic | Show Distance on OLED |

All values will appear on the OLED screen.

# 🔌 Pin Connections

### OLED (I2C)

| OLED | Arduino |
| --- | --- |
| GND | GND |
| VCC | 5V |
| SCL | A5 |
| SDA | A4 |

### Sensors

| Sensor | Pin |
| --- | --- |
| MQ2 | A0 |
| LDR | A1 |
| Rain Sensor | D3 |
| IR Motion | D4 |
| Buzzer | D5 |
| Light LED | D6 |
| Servo | D7 |
| DHT11 | D2 |
| Ultrasonic Trig | D9 |
| Ultrasonic Echo | D10 |

# 📦 Required Libraries

Install in Arduino IDE:

- Adafruit SSD1306

- Adafruit GFX

- DHT sensor library

- Servo library

# 💻 Complete Smart Home Arduino Code

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <DHT.h>

#include <Servo.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

Servo windowServo;

int gasPin = A0;

int ldrPin = A1;

int rainPin = 3;

int motionPin = 4;

int buzzer = 5;

int lightLED = 6;

int servoPin = 7;

int trig = 9;

int echo = 10;

long duration;

int distance;

void setup() {

Serial.begin(9600);

pinMode(rainPin, INPUT);

pinMode(motionPin, INPUT);

pinMode(buzzer, OUTPUT);

pinMode(lightLED, OUTPUT);

pinMode(trig, OUTPUT);

pinMode(echo, INPUT);

windowServo.attach(servoPin);

dht.begin();

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

}

void loop() {

float temp = dht.readTemperature();

float hum = dht.readHumidity();

int gasValue = analogRead(gasPin);

int ldrValue = analogRead(ldrPin);

int rainState = digitalRead(rainPin);

int motionState = digitalRead(motionPin);

digitalWrite(trig, LOW);

delayMicroseconds(2);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

duration = pulseIn(echo, HIGH);

distance = duration * 0.034 / 2;

if(gasValue > 400)

{

digitalWrite(buzzer, HIGH);

}

else

{

digitalWrite(buzzer, LOW);

}

if(motionState == HIGH)

{

digitalWrite(lightLED, HIGH);

}

if(rainState == LOW)

{

windowServo.write(0);

}

else

{

windowServo.write(90);

}

if(ldrValue < 300)

{

digitalWrite(lightLED, HIGH);

}

else

{

digitalWrite(lightLED, LOW);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

display.setCursor(0,0);

display.print("Temp: ");

display.print(temp);

display.setCursor(0,10);

display.print("Hum: ");

display.print(hum);

display.setCursor(0,20);

display.print("Gas: ");

display.print(gasValue);

display.setCursor(0,30);

display.print("Dist: ");

display.print(distance);

display.setCursor(0,40);

display.print("LDR: ");

display.print(ldrValue);

display.display();

delay(1000);

}

# 🖥 OLED Display Example

Temp: 29

Hum: 65

Gas: 250

Dist: 40

LDR: 320

# 🚀 Project Name Ideas (for presentation)

1️⃣ Smart Home Safety & Automation System
2️⃣ IoT Based Smart Home Monitoring
3️⃣ Multi Sensor Smart Home Using Arduino

✅ If you want, I can also give you:

- Full Circuit Diagram (very important for project)

- Better OLED interface (professional look)

- Add Mobile Control using Bluetooth / WiFi

- Project report for college submission

Just tell me and I will prepare it for you.