A Smart Entry–Exit Counting System with LCD (I2C) counts how many people enter and leave a room and shows the current number on a 16×2 LCD display. Usually two sensors are used:

- Sensor-1 → Entry (+1 count)

- Sensor-2 → Exit (−1 count)

The Arduino reads both sensors and updates the count on the LCD in real time. (ElectroDuino)

# Smart Entry Exit Counter (Arduino + LCD I2C)

## 🧰 Components

- Arduino Uno / Nano

- 2 × Ultrasonic sensor (HC-SR04) or IR sensor

- 16×2 LCD

- I2C module for LCD

- Breadboard

- Jumper wires

LCD with I2C uses only 4 wires (VCC, GND, SDA, SCL) which simplifies wiring. (halina20011.github.io)

# 🔌 Connections

## 1️⃣ LCD with I2C

| LCD I2C | Arduino |
| --- | --- |
| GND | GND |
| VCC | 5V |
| SDA | A4 |
| SCL | A5 |

## 2️⃣ Ultrasonic Sensor 1 (Entry)

| Sensor Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | 8 |
| ECHO | 9 |

## 3️⃣ Ultrasonic Sensor 2 (Exit)

| Sensor Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | 10 |
| ECHO | 11 |

# 💻 Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27,16,2);

int trig1 = 8;

int echo1 = 9;

int trig2 = 10;

int echo2 = 11;

long duration;

int distance;

int count = 0;

void setup() {

lcd.init();

lcd.backlight();

pinMode(trig1, OUTPUT);

pinMode(echo1, INPUT);

pinMode(trig2, OUTPUT);

pinMode(echo2, INPUT);

lcd.setCursor(0,0);

lcd.print("Visitor Counter");

}

int getDistance(int trig,int echo){

digitalWrite(trig, LOW);

delayMicroseconds(2);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

duration = pulseIn(echo,HIGH);

distance = duration * 0.034 / 2;

return distance;

}

void loop() {

int d1 = getDistance(trig1,echo1);

int d2 = getDistance(trig2,echo2);

if(d1 < 20){

count++;

delay(1000);

}

if(d2 < 20 && count > 0){

count--;

delay(1000);

}

lcd.setCursor(0,1);

lcd.print("People: ");

lcd.print(count);

lcd.print("   ");

}

# ⚙️ Working

- Entry sensor detects person → count +1

- Exit sensor detects person → count −1

- Arduino updates LCD instantly.

- LCD shows total people inside the room. (ElectroDuino)

✅ Example LCD Display

Visitor Counter

People: 3

💡 Since you already asked about two ultrasonic sensors yesterday, I can also give you a better professional version (100% accurate entry-exit logic used in malls) with a sequence detection algorithm.

If you want, I can also show:

- 📷 Complete circuit diagram

- 🔧 Tinkercad simulation

- 🧠 Advanced code (no double counting).