Here is a Turbidity Sensor + Arduino + 16×2 LCD (I2C) Project. This will show water quality directly on the LCD, which is great for exhibitions or demos.

## 🔧 Components Required

- Arduino Uno

- Turbidity Sensor Module

- 16×2 LCD Display

- I2C Module (for LCD)

- Jumper Wires

## 🔌 Connections

### 1️⃣ Turbidity Sensor → Arduino

| Sensor Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| AO | A0 |

### 2️⃣ LCD (I2C) → Arduino

| I2C LCD Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

## 💻 Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27,16,2);

int sensorPin = A0;

int turbidity;

void setup()

{

lcd.begin();

lcd.backlight();

Serial.begin(9600);

}

void loop()

{

turbidity = analogRead(sensorPin);

lcd.clear();

lcd.setCursor(0,0);

lcd.print("Turbidity:");

lcd.print(turbidity);

lcd.setCursor(0,1);

if(turbidity > 700)

{

lcd.print("Water: Dirty");

}

else

{

lcd.print("Water: Clean");

}

delay(1000);

}

## ⚙️ Working

- Turbidity sensor measures water clarity.

- Arduino reads analog value from A0.

- If value > 700 → Dirty Water.

- If value < 700 → Clean Water.

- Result appears on 16×2 LCD.

Example LCD Display:

Turbidity: 820

Water: Dirty

✅ Since you already used LCD I2C in your smart counting project, this wiring will be exactly the same.

💡 If you want, I can also show you a more advanced Water Quality Monitoring Project using:

- Turbidity Sensor

- pH Sensor

- Temperature Sensor

- LCD Display

This becomes a very strong BCA/engineering level project.