Your OLED pins (GND, VDD, SCK, SDA) indicate it is an I2C OLED display (SSD1306). The connection with Arduino Uno is very simple.

## 🔌 OLED → Arduino Connection

| OLED Pin | Arduino Uno Pin |
| --- | --- |
| GND | GND |
| VDD | 5V |
| SCK | A5 |
| SDA | A4 |

⚠️ Note:

- SCK = SCL (Clock)

- Many OLED modules write SCK instead of SCL.

## 🔌 Turbidity Sensor → Arduino

| Sensor Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| AO | A0 |

## 📊 Full Connection Summary

OLED GND → Arduino GND

OLED VDD → Arduino 5V

OLED SCK → Arduino A5

OLED SDA → Arduino A4

Turbidity VCC → Arduino 5V

Turbidity GND → Arduino GND

Turbidity AO  → Arduino A0

## 💻 Arduino Code (OLED + Turbidity)

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(128, 64, &Wire, -1);

int sensorPin = A0;

int turbidity;

void setup() {

Serial.begin(9600);

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED not found");

while(true);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

}

void loop() {

turbidity = analogRead(sensorPin);

display.clearDisplay();

display.setCursor(0,10);

display.print("Turbidity:");

display.println(turbidity);

display.setCursor(0,30);

if(turbidity > 700)

display.println("Water Dirty");

else

display.println("Water Clean");

display.display();

delay(1000);

}

✅ Tip:
If the OLED does not show anything, check the I2C address (0x3C or 0x3D).

If you want, I can also show you a very advanced Arduino project using this sensor:

⭐ Smart Water Quality Monitor with OLED + Buzzer + RGB LED

This looks very impressive in science exhibition / BCA project.